Certification and Reporting (CG-500) for Controls and Safety Devices

Unit Manufacturer

Name________LOCHINVAR CORPORATION

Address_____  300 MADDOX SIMPSON PARKWAY /   LEBANON, TN
 Zip ___37090

Telephone____615-889-8900
   Fax____615-547-1000


Boiler Identification 

Manufacturer's Model #   KBN 501
  Year Built


ASME #     34,384
  Nat. Bd. #


AGA #   189136-1686154_______________________Jurisdiction          USA & Canada

Hot Water
Max. W.P. _160____
 psig  Max. Temp. _200___
 ºF

Min. Safety Relief Valve Cap.          697,000

        Btu

Boiler Unit Description (Type)    Low Pressure Boiler


Boiler Unit Capacity (Output)              467,000  Btu/hr    


Burner

Manufacturer's      Bekaert 
  Model      221.049.001-A 


UL or AGA #     189136-1686154
  Serial #____N/A


Fuels (as Shipped)      Natural Gas


                                                                                                Indicate Units  (Where not applicable, indicate “N/A”)

    Gas Manifold Pressure                                                     __________________N/A_______________________                                                                                     

    High Gas Pressure Switch Setting                                    _____________11.0” w.c. / gross_________________                                                                                             

Installation Location (if Known)

Customer Name 

Address 

City 
  State 
Zip 

Telephone
   Fax





Operational Test

Control/Device
Manufacturer
Model #
Performed, Date


Operating Controls

Low-Water Fuel Cutoff


CW-120(a), CW-140
    Hydrolevel  (opt.)

          45-506

            


Forced Circulation


CW-210(a) 
            TACO     

      IFSWSBL-S4        



Water Temperature


CW-410(b) 
       SIT Controls   

     BIC-935/C



Safety Controls

Low-Water Fuel Cutoff


CW-120(a), CW-120(b)


CS-130, CW-140
   Hydrolevel  (opt.)

           45-506

            

Forced Circulation


CW-210(b) 
            TACO  

       ISFWSBL-S4   

            

High Water Temperature Limit


CW-410(b) 
          Tasseron

        TSD91A0



Fuel Safety Shutoff Valve,

Main


CF-180(b)(2), CF-180(b)(3) 
          Dungs   

 GB-WND 057 D01 S02-XP

Combustion Air Switch


CF-220
 Cleveland Controls

     ANA-200-351         
                 


High Gas Pressure


CF-162
       Honeywell  (opt.)_
        C6097B-1012



Low Gas Pressure


CF-162
       Honeywell (opt.)  

        C6097B-1028



Flame Safeguard (Primary)


CF-310, CF 320
        SIT Controls

         BIC-935/C




Flame Detector


CF-310, CF-320
         Rauschert

        48149M03



Safety or Safety Relief Valve(s)


CW-510, CW-520
    Apollo/Conbraco

         10-417-10 



(opt.) = Optional Components that may not be installed on this appliance.  The installer may strike through an item if it is not on the appliance.

Manufacturer   LOCHINVAR CORPORATION

Operational Test Performed, Date 
/
/

Model               KBN 501


Size                 500,000                                       Btu/hr

Capacity         467,000
         Btu/hr

Representing Equipment Manufacturer, Name 

Signature 

Date 

Representing Installing Contractor, Name 

Signature 

Date 


